ORGANISATIONAL PERFORMANCE MEASUREMENT OF NIGERIAN INDIGENOUS CONTRACTORS USING KEY PERFORMANCE INDICATORS

Abdulkadir Shehu RASHEED1, Ranti Taibat ADEBIYI1, Theophilus O.OLOWA1 and Nofiu Abiodun MUSA2

Department of Quantity Surveying, University of Ilorin, Ilorin, Kwara State, Nigeria

Department of Quantity Surveying, ObafemiAwolowo University, Ile-Ife, State of Osun, Nigeria

Corresponding Author: rasheed.as@unilorin.edu.ng, Tel: +234-809-155-1476

ABSTRACT

Measuring organisational performance of construction firms with Key Performance Indicators (KPIs) at project level has become inadequate hence KPIs measured at organisational level are necessary and already relevant to contracting firms in several countries. Nigerian Indigenous Contractors (NICs) are still largely assessed through project level KPIs and this has contributed to below average organizational performance resulting in close down of many firms and difficulties in delivering complex infrastructure projects. This study critically examines KPIs at organizational level of NICs. Review of literature synthesized 75 KPIs. Factor analysis was used to reduce the KPIs to 53 measuring 6 factors. Structured questionnaire was administered to 486 construction professionals comprising Architects, Quantity Surveyors, Engineers and Builders in management positions of NICs from Abuja, Lagos and Rivers, 256 was returned representing 53% response rate. Factors were ranked using mean and standard deviations. Procurement factors ranked highest amongst factors measured by the KPIs with an average mean of 5.98, business procedure and finance ranked equally high at 5.94 and 5.79 respectively. Other factors measurable with the KPIs include organisational knowledge and project management factors. Environmental factors ranked lowest at 4.41. The study identified latent KPIs relevant to measurement of organisational performance factors in NICs. Measurable factors are important indices for adequate assessment NICs performance at the organisational level. It is recommended that NICs inculcate these measurable indices for adequate assessment of their organisational performance.

Keywords: Construction Professionals; Key Performance Indicators; Nigerian Indigenous Contractor; Organisational Performance

- INTRODUCTION

The last two decades have seen considerable research into the development and implementation of Key Performance Indicators (KPIs) for different types of construction works (Sohail and Baldwin, 2004). Researchers have deviated from the traditional iron triangle of time, cost and specification (Toor and Ogunlana, 2010). For example, one fundamental deviation is the study by Chan and Chan (2004) that separated KPIs into 8 objective measures and 6 subjective measures. Construction time, speed of construction, time variation, unit costs, percentage net variation over final cost, net present value, accident rate and Environment Impact Assessment (EIA) scores were listed as objective indicators and subjective indicators listed are quality, functionality, end user's satisfaction, client’s satisfaction, design team’s satisfaction and construction teams’ satisfaction. Nitithamyong and Skibniewski, (2006) looked at KPIs from a broader perspective they considered; strategic plan, risk, and communication as key indicators with measurable indices in addition to the KPIs in the traditional iron triangle. Similarly, stakeholder satisfaction was included as a KPI in a study of the performance of project plans (Idoro, 2009). Toor and Ogunlana, (2010) also posited that safety, efficient use of resources, effectiveness, reduced conflicts, and disputes are increasingly becoming important as KPIs on large public sector development projects. Furthermore, in a study of the sustainability of green buildings Konstantinos et al., (2012) identified 7 indicators to include: reduction of Waste; Customer Satisfaction; Education of Personnel; Capacity Utilisation; Service and Quality; Client Cooperation; and Use of Technology as KPIs. However, these are performance measurements concentrated at the project level and do not consider measurements at the organizational level of construction firms.

Research into KPIs in more recent times has improved on performance evaluation areas to include not only the indicators that measures at project level but also the firms, stakeholders’ management and human resource levels (Gudiene et al., 2013; Ali et al., 2013; Alias et al., 2014; Wibowo et al., 2015; Goodenough et al., 2017, Hidiroglu, 2019). Similarly, performance measurement of organizational effectiveness (both financial and non-financial dimensions) has been identified to be a priority contributing to project success (Hidiroglu, 2019). This area of research is however still limited and country-specific with developed KPIs that are suitable for specific national construction sectors (Toor and Ogunlana, 2010; Landstrom et al., 2018). In addition, Alias et al., (2014) averred that there is no comprehensive study that explores the important critical success factors from the perspective of project management practitioners. Thereby, suggesting a comprehensive investigation about the perception of the practitioner on the effect of the critical success factors on performance at the organization level.

Developing construction firms’ capacity is an important issue concerning their performance in the industry. The Nigerian Indigenous Contractor (NIC) has faced many challenges in the industry. For example, they have been plagued with below-average performances in the delivery of large building, civil and heavy engineering construction projects (Adams, 1997; Chen, et al., 2007; Ogbebor, 2014). Other significant challenges include the lack of local skilled labor, power shortage, the unavailability of materials, and the unethical practices that are very common in the industry (Sanusi 2008). Performance indicators designed in different sections of construction firms for a combined objective of achieving project success lack an integrated way of measurement and monitoring (Fernanda and Milton, 2017). Also in a study of Business Performance Measurement Systems (BPMS), Landstrom et al., (2018) identified that one primary pitfall is that KPIs are not clearly connected to the strategic objectives and goals of the organisation which makes it harder to understand the purpose of the specific KPI and why it is important for the organization. Considering the ever-rising competition in the construction market (Wibowo et al., 2015), firms need to monitor their progress constructively through appropriate indicators to maintain their competitiveness to survive in national and international markets.

Advancing appropriate strategies for the improvement of contractors’ performances through the prioritization of their KPIs will ultimately improve the categorization ratings of NICs as well as reduce the use of expatriate firms. The aim of this study is to critically examine Key Performance Indicators (KPIs) applicable to the Nigerian Indigenous Contractor (NICs) at the organizational level, with a view to identifying the more important indicators to measure their organizational performance.

- ORGANISATIONAL PERFORMANCE MEASUREMENT SYSTEM

Performance measurement system is an important process that quantifies efficiency and effectiveness (Wibowo et al., 2015). This measurement system evaluates the operation of organizations to reveal their performance and for projects to reveal its success. Ali et al., (2013) citing Cokins, (2006) posited that the main objective of performance evaluation is to assist managers and members of the organization to develop the direction, traction and speed of their organization. Therefore, the organizational performance measurement system is used as information to deploy policy and strategy and also to obtain feedback (Wibowo et al., 2015). The current competition into global construction markets has necessitated companies to monitor their practices and results to ensure competitiveness. To survive these challenges and compete successfully, organizations need to monitor processes through KPIs (Antunes da Silva and Borsato, 2017).

  - Performance Indicator Perspectives

Several studies looked at performance indicators outside the project level as it affects different aspects of the constructing firm. The perspectives for deriving measurement factors are similar but with very distinct exceptions.

Ali et al., (2013) identified forty-seven (47) indicators that were grouped into four (4) factors of financial, customer, internal business, and environment. Mohamad et al., (2013) in their study of financial factors posited that a key financial performance indicator is the net-profit of the constructing firm because it is the primary goal of any firm. A firm’s ability to compete and be successful in a project is a unit of the performances expected in the firm as the project success also contributes to the firms’ overall success. They harmonized twenty three (23) factors affecting the company’s net profit into two limited factors of external (firm related factors) and external (market related factors). Wibowo et al., (2015) also categorized bidding strategy as an important performance indicator at the organizational level where five (5) factors of: financial; internal business processes; learning and growth; customer satisfaction; and environment were identified as performance measures on the organizational level. Tripathi and Jha (2017) identified twenty (20) organizational performance attributes which resulted in six (6) performance factors of: profitability and asset management; satisfaction of key stakeholders; predictability of time and cost; environment, health, and safety (EHS); quality consciousness; and low staff turnover.  This is the first study on KPIs at organizational level which focused on a developing economy.

In a different perspective, Alias et al., (2014) looked at Critical Success Factors (CSFs) in project management as inputs to project management practice which can lead directly or indirectly to project success. Twenty - two (22) indicators were reduced into five (5) perspectives of project management action; project-related factors; project procedures; external issues; and human factors. The importance of CSFs is that they are useful in decision-making which can significantly affect the performance of projects. CSFs can be used as performance indicators for the constructing firm as the concept of CSFs presents a smarter way to identify certain factors which when present or absent in a project are likely to make the project successful (Gudiene et al., 2013). Similarly, the negative impact of construction activities on the environment has necessitated sustainability studies around the world. The United Nations development agenda created the Sustainable Development Goals (SDGs) with two (2) of the key areas being to build resilient infrastructure, promote inclusive and sustainable industrialization and foster innovation; and to make cities and human settlements inclusive, safe, resilient and sustainable (Gaffney, 2014; Osborn, 2015). This global development agenda made construction organisations’ performances include sustainability measures. Afzal et al., (2017) reported fifteen (15) sustainability performance indicators adopted by the Global Reporting Initiative (GRI) reduced to three (3) factors of economic, social and environmental indicators. Sustainability has become a cogent requirement for constructing firms to abide with if they are to compete globally in any regard.

  - Identifying Key Performance Indicators at the Organizational Level

In the aggressive and competitive atmosphere existing in the construction industry, contracting firms requires constant self-checks to ascertain their performance. Business performance is usually measured in quantitative terms of net profits and total turnover, however contracting businesses rely also on qualitative measures like managers competences and employee expertise to survive (Tripathi and Jha, 2017 citing Cox et al., 2003).

Lin et al., (2011) outlined steps required towards the measurement of performance of the construction organizations as: (i) Identification of appropriate Key Performance Indicators (KPIs) and (ii) employing the identified sets of KPIs in developing a proper performance measurement framework for construction organizations. This study proposes to identify the more indicated Key Performance Indicators on the organizational level of contracting firms in Nigeria. Relevant indicators drawn from literature are as summarized in table 1 below.

Table 1: Key Performance Indicators as studied in Previous Literature

| Authors and Year | KPI Perspective (Constructs) | Key Performance Indicators (Measurements) |  |
| --- | --- | --- | --- |
| Mohammad et al., (2013) | Firm Related Financial Factors | 1.Business Size | 9.Working Capital Policy |
|  |  | 2.Sales Growth | 10.Inventory of Turnover Rate |
|  |  | 3.Operating Cash Flow | 11.Inventory Management Efficiency |
|  |  | 4.Return On Assets | 12. Receivables Management. |
|  |  | 5.Leverage | 13.Liabilities Management |
|  |  | 6.Firm Debt Ratio | 14.Tangible Assets Ratio |
|  |  | 7.Firm Market Power | 15.Average Collection Period |
|  |  | 8.Cash Conversion Cycle | 16.Average Payment Period |
|  | Market Related Financial Factors | 1.Level of Economic Activity | 5.Demand on Construction |
|  |  | 2.Market Performance | 6.Competition |
|  |  | 3.Inflation Rate | 7.Cost of External Financing |
|  |  | 4.Financial Distress |  |
| Alias et al., (2014) | Project Management Actions Factors | 1.Communication System | 4.Effective Safety Programme |
|  |  | 2.Planning Effort | 5.Quality Assurance programme |
|  |  | 3.Organization Structure, | 6.Sub-Contractor Management |
|  | Project Procedures Factors | 1.Procurement Method | 2.Tendering Strategy |
|  | Human Factors | 1.Client's Experience | 4.Client's Project Philosophy |
|  |  | 2.Nature of Client | 5.Client Decision & Briefing Skills |
|  |  | 3.Size of Client's Organization |  |
|  | Project Related Factors | 1.Project Type | 3.Project Complexity |
|  |  | 2.Project Nature | 4.Project Size |
|  | External Factors | 1.Economic Influences | 4.Physical Influences |
|  |  | 2.Social Influences | 5.Technology Advancement |
|  |  | 3.Political Influences |  |
| Afzal et al., (2017) | Economic Sustainability Factor | 1. Financial Performance | 3.Indirect Economic Impacts |
|  |  | 2. Market Presence |  |
|  | Social Sustainability Factors | 1. Community. | 3. Labour practice & Decent Work. |
|  |  | 2. Human rights | 4.Product responsibility |
|  | Environmental Sustainability Factors | 1.Materials | 5.Emissions and waste |
|  |  | 2.Energy | 6.Products and service |
|  |  | 3.Water | 7.Compliance |
|  |  | 4.Biodiversity | 8.Transport |
| Ali et al., (2013) | Financial | 1.Profitability | 5.Financial Performance |
|  |  | 2.Growth | 6.Capital |
|  |  | 3.Financial Stability | 7. Invest. in Dev. of New Markets |
|  |  | 4.Cash flow | 8. Interest Cover |
|  | Customer | 1.Quality of Service and Work | 5.Number of New Customers |
|  |  | 2.Value for Money | 6.External Customer Satisfaction |
|  |  | 3.Market Share | 7.Competitive Price |
|  |  | 4. Hassle-free Relationship | 8.Internal Customer Satisfaction |
|  | Internal Business | 1.Safety | 9.Resource Management |
|  |  | 2.Business Efficiency | 10.Technological Capability |
|  |  | 3.Effectiveness of Planning | 11.Productivity |
|  |  | 4.Labor Efficiency | 12.Quality Control and Rework |
|  |  | 5.Successful Tenders Rate | 13.Defects |
|  |  | 6.Managers Competency | 14.Research and Development |
|  |  | 7.Innovation | 15.Staff Turnover |
|  |  | 8. Number of high Performance Professionals |  |
|  | Learning and growth | 1.Motivation | 5.HR Training & Development |
|  |  | 2.Continuous Improvement | 6.Informatization |
|  |  | 3.Investors in People | 7.Empowered Work Force |
|  |  | 4. Organization Competency in Management of HR |  |
|  | Environment | 1.Risk control | 6.Waste |
|  |  | 2.Partnership and suppliers | 7.Energy use |
|  |  | 3.Policy or law of government | 8.Main water use |
|  |  | 4.Competitors | 9.Impact on biodiversity |
|  |  | 5.Impact on society |  |

  - Research Concept and Framework

This study will follow a research framework as shown below:

Figure 1: Research Framework for the Study

Literature Review

One Hundred and Seven (107) KPIs

Identify initial KPIs from previous studies

Methodology				Purpose				Outcome

Remove repeated KPIs & streamline Factors

First Version of Questionnaire

,,

Synthesis of KPIs

Final Version of Questionnaire

To identify & remedy errors in initial instrument

Pre-testing initial Questionnaire

Revised KPIs to measure performance

Prioritizing KPIs

Survey distribution and data gathering

It was observed from literature that many of the constructs (indicators) overlap and measure similar factors. KPIs that are similar to each other and are under the same factor are merged in this study. An example is ‘working capital’ (Mohammad et al., 2013) and ‘capital’ (Ali et al., 2013) which are both under ‘financials’ factors.  In the same light, some other constructs measure similar results but have been categorized under different factors, these type of overlap have been synthesized into one for this study. An example is ‘profitability’ and ‘financial stability’ both measuring the organizations quantitative success in turnover grouped under ‘financial’ (Ali et al., 2013) and ‘economic sustainability’ (Afzal et al., 2017) factors respectively have been merged. Subsequently, seventy-five (75) Key Performance Indicators (KPIs) are used to measure eight (8) factors which directly affect performance at the organisation level of contracting firms.  The KPIs will be examined for relevance to the factors that they measure and the relationship of the factors will also be examined to ensure they relate to measuring the organizational performance of construction firms in Nigeria.

Construction firm as used in this study will refer to Nigerian Indigenous Contracting firms that are fully-owned and managed by Nigerians; the nationality of the firms’ ownership and management is exclusively Nigerian (Inuwa et al., 2014).  A conceptual framework adopted for this study is presented in figure 2 below.

FACTORS

Figure 2: Conceptual Framework for the Study

1. Business size.	   7.   Firm debt ratio. 	         13. Cash conversion cycle.

2. Sales growth.	      	   8.   Market performance.      14. Level of economic activity.

3. Operating cash flow.	   9.   Inflation rate.	         15. Demand on construction.

4. Return on assets.	  10.  Financial distress.          16. Cost of external financing.

5. Leverage.		  11.  Competition.	         17. Working capital policy.

6. Firm market power.	  12.  Profitability. 	         18. Financial stability.

KEY PERFORMANCE INDICICATORS MEASURED

Finance & Economic

Factors

PM & Internal Business Procedure Factors

1. Communication system    7.   Innovations.	           13. Organizational Structure.

2. Business efficiency.	       8.   Procurement Method.  14. Planning effort & effect.

3. Technology capability.      9.   Labour efficiency.       15. Manger & staff competence.

4. Tender strategy.	      10.  Resource Mangt.          16. Effective safety programme.

5. Productivity.		      11.  Rework & defects frequency.

6. Sub-contractor Mangt.     12.  Quality Assurance programme.

1. Economic influences.        4.  Community effects.	      7. Labour practices &

2. Human rights.	       5.  Physical influences. 	          Decent work

3. Political influences.	       6.  Technology advancement.        .

1. Materials use.	       5. Main water use.	       9. Impact on biodiversity.

2. Risk control.		       6. Impact on society.    10. Transportation.

3. Partnerships & Suppliers. 7. Emission & waste.    11. Fulfilling Gov. policies & laws

4. Energy use.	       	       8. Product & service responsibility.

Environ.  Sustain.

Factors

External & Social Factors

Project Factors

1. Clients’ experience.	       3.   Clients’ project philosophy.         5. Size of clients org.

2. Nature of Client.	       4.   Clients’ decision & briefing skills.

1. Project type.		       3.   Project size.

2. Project nature.	       4.   Project complexity.

ORGANISATIONAL PERFORMANCE MEASUREMENT

Human Factors

Customer Satisfaction Factors

Learning     &Growth Factors

1. Competitive price.             4.  Quality of service & work.    7. Hassle-free relationship.

2. Value for money.      	       5.  Customer satisfaction.

3. Market share.	            	       6.  Number of new customers.

1. Continuous improvement.  4. Informatisation.	                         7. HR Training & Dev.

2. Investments in people.        5.  Empowered workforce.

3. Motivation.		         6. Org. competence in managing HR.

- RESEARCH METHODOLOGY

A Cross-sectional survey approach was adopted for the study employing a questionnaire administered using Google forms sent to e-mails and registered phone numbers of construction professionals comprising architects, quantity surveyors, engineers, and builders who are in management positions in contracting organizations. Managers are required for the study to ensure reliability of responses as they affect the organization. The questions are designed to elicit responses on the significance of the KPIs in measuring organizational level performance and are rated on a 7 point Likert scale ranging from Very Low (VL) = 1 to Very High (VH) = 7. The population was drawn from a list of 1,669 building & civil engineering contracting firms registered on Vconnect an online directory of businesses in Nigeria. The study received responses from contracting firms with registered head offices in Lagos, Rivers and Abuja. These location strata represent areas of high construction activities in Nigeria. In order to achieve a robust estimate within a substantively important stratum, the disproportionate stratification sampling was adopted.  With this method the sample size of each stratum is not proportionate to the population size of the stratum (Kalton, 2001; Explorable, 2009). This method allows adequate sampling within concentrated strata, sampling of rare extremes and also optimizing statistical efficiency (Karlton, 2001; Sturgis, 2004). IBM SPSS Statistics 25 was employed to analyze data received. First a test of reliability of construct was carried out to elicit the alpha values of the Key Performance Indicators (KPIs). An Exploratory Factor Analysis (EFA) was then used to confirm the groupings of the KPIs. Principal Component Analysis (PCA) for factor extraction was applied to reduce the KPI’s into fewer numbers. To examine the appropriateness of the data collected for factor analysis, the Kaiser–Meyer–Olkin (KMO) measure of sampling adequacy and Bartlett’s test of sphericity were computed. Finally, the factors were ranked based on their mean and standard deviation to check internal consistency of responses. A comparison of the rankings was done between the respondent groups.

  - Determination of Sample Size and Test of Adequacy

Due to unequal sizes of population, disproportionate sampling was employed using sample fractions of population in each stratum in relation to total population. The minimum sample size required was derived using the formula (Ali et al., 2013 citing Kish, 1995) below:

n1 							(p x q)

n = 	(1 + n1/N)	……….	(1);		n1 = 	    V2 	      ………….		(2)

Where:	n = the required sample size; n1 = first estimate of sample size; N =	 population size;

p = proportion of the characteristics being measured in stratum; q =	1 – p and;

V = standard error of sampling population (= 0.10 for this study).

Out of 486 questionnaires distributed, 256 was returned representing 53% response rate. However only 244 (50%) of the responses received was suitable for analysis and used for the study since this is above the minimum required sample of 243. The Summary of responses according to the strata is shown in Table 2 below.

Table 2:    Location and response rate of respondents for the survey

| Location (States) | Population (No.) | Proportion (p) | Req. Sample (n) | Questionnaires Distributed | Response used (No.) (%) | Adequacy |
| --- | --- | --- | --- | --- | --- | --- |
| Lagos | 740 | 44% | 93 | 186 | 97 52 | > Min. |
| Abuja | 349 | 21% | 64 | 128 | 59 46 | < Min. |
| Rivers | 580 | 35% | 86 | 172 | 88 51 | > Min. |
| Total | 1669 | 100% | 243 | 486 | 244 50 | > Min. |

The test for adequacy utilized the Kaiser–Meyer–Olkin (KMO) measure of sampling adequacy as averred by Field (2005) that values between 0.5 and 0.7 are mediocre, and values between 0.7 and 0.8 are good. In addition, Bartlett’s test of sphericity which test the presence of correlation among constructs requiring p < 0.005 (Hair et al., 2006) was carried out.

Table 3:   KMO and Bartlett’s Test for Sample Adequacy

| KMO and Bartlett’s Test Kaiser-Meyer-Olkin Measure of Sampling Adequacy |  | .813 |
| --- | --- | --- |
| Bartlett's Test of Sphericity | Approx.Chi.Square | 32770.626 |
|  | df | 2775 |
|  | Sig. | .000 |

Table 3 shows results of the KMO test at 0.809 and Bartlett’s test of sphericity at p < 0.005.

  - Reliability and Validity of Construct

The study utilized Cronbach’s Alpha for test of reliability of construct. Table 4 shows that alpha values for each factor is greater than 0.70. The alpha reflects the extent to which the different KPIs tested produced similar measures and signifying a high relationship between the factors they measured (Taber, 2018). Elimination of construct using Exploratory Factor Analysis (EFA) requires high factor loadings to consider constructs. Hair et al, (2006) considered factor loadings of 0.5 or greater as practically significant, hence this study only considered factor loading equal to or greater than 0.5. Furthermore items loading on more than one factor that is, with a loading score equal to or greater than 0.4 on more than one factor was dropped from the analysis since cross loading variables make interpreting the factor difficult (Hair et al., 2006).

- RESULTS

In terms of location of respondents, 40% of the respondents are professionals managing NICs in the Lagos area, 24% from Abuja and 36% from Rivers state. Results shows that about 57% of the respondents are in top management positions of the organizations sampled, 33% are middle managers and 10% in lower management positions. Their educational qualification revealed that 12% had PhD degree 59% had MSc degree, 28% had HND/BSc degree and 1% Diplomas. A large percentage of the respondents (69%) have work experience above 20 years, 11% ranged between 16-20 years, 12% between 10-15 years and 8% between 5-9 years. About 26% of the respondents are Architects, 32% Quantity Surveyors, 35% Engineers while 7% are Builders.

  - Exploratory Factor Analysis (EFA)

Exploratory factor analysis is primarily used to reduce data to a smaller set of summary variables. In addition, it is also used to explore and identify the structure of the relationship between the variables and what they measure (Hair et al., 2006). It assesses whether items should be eliminated from the scale because they have obtained data or are loaded in more than one factor (Field, 2005; Hair et al., 2006 & Taber, 2018). This study considered factor loadings of 0.5 or greater as practically significant.

Based on the eight (8) factor groupings shown in the framework (Figure 2), the analysis utilizing Varimax rotation with Kaiser Normalization and extraction of principal components with factor groupings having eigenvalues greater than 1.0 explains 69.782% of the variance. In addition, the results revealed some constructs do not exhibit a simple structure. This means that they are loaded weakly on more than one factor grouping or loaded weakly with their synthesized factor grouping. In the study, 16 factors displayed one or both characteristics which includes: eight (8) KPIs measuring financial and economic factors, three (3) KPIs measuring external and social factors and all five (5) KPIs measuring human factors.

Subsequently the 59 KPIs retained after removing the 16 with unusual characteristics was subjected to a principal component analysis with Varimax rotation to examine the factor structure of the remaining measures. The scree plot indicated a convergence in six (6) iterations hence, 53 factors have emerged with eigenvalues above 1.0, explaining 75.542% of the variance in the data, as can be seen in Table 4.

Table 4:   Results of Principal Component Analysis.

| FACTORS | CONSTRUCT (KPIs) | F1 | F2 | F3 | F4 | F5 | F6 | Alpha |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FINANCIAL (FIRM & MARKET) FACTOR |  |  |  |  |  |  |  | .989 |
|  | Firm debt ratio | .968 | -.092 | -.023 | .009 | -.110 | .024 |  |
|  | Profitability | .968 | -.088 | -.030 | .010 | -.109 | .014 |  |
|  | Cost of external financing | .967 | -.084 | -.030 | .003 | -.113 | .009 |  |
|  | Level of economic activity | .965 | -.079 | -.021 | .002 | -.110 | .001 |  |
|  | Leverage | .963 | -.066 | -.037 | .001 | -.103 | -.008 |  |
|  | Competition | .961 | -.102 | -.033 | .011 | -.105 | .029 |  |
|  | Working capital policy | .958 | -.080 | -.024 | -.005 | -.119 | .000 |  |
|  | Customer satisfaction | .948 | -.069 | .016 | -.010 | -.037 | .027 |  |
|  | Business size | .947 | -.032 | .020 | -.032 | -.047 | -.007 |  |
|  | Operating cashflow | .941 | -.094 | -.036 | .011 | -.104 | .046 |  |
|  | Market share | .939 | -.074 | .004 | .008 | -.060 | -.006 |  |
|  | Competitive price | .933 | -.094 | -.010 | -.032 | -.052 | .001 |  |
|  | Quality of service and work | .922 | -.132 | -.057 | .002 | -.113 | .041 |  |
|  | Value for money | .907 | -.107 | -.035 | .020 | -.034 | .053 |  |
|  | Inflation rate | .876 | -.089 | .003 | -.054 | -.071 | .049 |  |
|  | Empowered workforce | .851 | -.005 | .075 | -.077 | -.026 | -.010 |  |
|  | Informatisation | .807 | -.066 | -.005 | -.066 | -.107 | .044 |  |
|  | Continuous improvement | .807 | -.038 | .061 | -.024 | -.060 | .021 |  |
|  | Motivation | .745 | .014 | .102 | -.031 | .022 | -.013 |  |
|  | Investment in people | .726 | -.004 | .077 | -.009 | .021 | .033 |  |
| ENVIRONMENTAL FACTOR |  |  |  |  |  |  |  | .980 |
|  | Partnership and suppliers | -.080 | .974 | .017 | .028 | .054 | .046 |  |
|  | Project type | -.076 | .972 | .017 | .032 | .052 | .055 |  |
|  | Material use | -.073 | .955 | .023 | .016 | .052 | .044 |  |
|  | Human rights of third party | -.109 | .941 | .018 | .026 | .051 | .043 |  |
|  | Risk control | -.045 | .938 | .013 | -.034 | .033 | .024 |  |
|  | Transportation | -.106 | .886 | .036 | .044 | .047 | .047 |  |
|  | Project complexity | -.075 | .884 | .025 | .047 | .065 | -.018 |  |
|  | Main water use | -.075 | .857 | .063 | -.022 | -.006 | .106 |  |
|  | Impact on society | -.088 | .849 | .001 | .068 | .084 | -.063 |  |
|  | Project size | -.082 | .848 | .047 | -.018 | -.013 | .100 |  |
|  | Emission and waste | -.105 | .843 | -.039 | -.011 | .060 | -.017 |  |
|  | Product & service responsibility | -.041 | .836 | .069 | -.044 | .056 | .061 |  |
|  | Project nature | -.058 | .832 | .044 | -.012 | .001 | .035 |  |
|  | Energy use | -.050 | .800 | .032 | -.017 | .007 | .020 |  |
|  | Fulfilling Gov. policies/laws | -.043 | .765 | .046 | .082 | .023 | .058 |  |
|  | Impact on biodiversity | -.031 | .699 | -.126 | .088 | .053 | -.097 |  |
| PROCUREMENT FACTOR |  |  |  |  |  |  |  | .985 |
|  | Procurement method | -.008 | .073 | .964 | .022 | .042 | -.076 |  |
|  | Effective safety programme | .012 | .056 | .963 | .003 | .035 | -.068 |  |
|  | Quality assurance programme | .014 | .065 | .954 | .006 | .034 | -.082 |  |
|  | Tender strategy | .018 | .052 | .954 | .030 | .037 | -.061 |  |
| PROJECT MANAGEMENT FACTOR |  |  |  |  |  |  |  | .882 |
|  | Organisational structure | -.044 | -.014 | .014 | .881 | -.171 | .117 |  |
|  | Communication system | -.061 | -.015 | .016 | .862 | -.190 | .099 |  |
|  | Productivity | -.018 | -.037 | .033 | .859 | -.176 | .123 |  |
|  | Labour efficiency | -.017 | -.037 | .026 | .849 | -.169 | .137 |  |
|  | Physical influences | -.018 | .225 | -.025 | .502 | .315 | -.336 |  |
| BUSINESS PROCEDURES FACTOR |  |  |  |  |  |  |  | .941 |
|  | Sub-contractor management | -.037 | -.055 | .164 | -.099 | .814 | .117 |  |
|  | Resource management | -.035 | -.052 | .166 | -.021 | .814 | .137 |  |
|  | Planning effort and effect | -.065 | -.031 | .187 | -.084 | .786 | .128 |  |
|  | Business efficiency | -.053 | -.013 | .079 | -.043 | .770 | .087 |  |
| ORGANISATIONAL KNOWLEDGE FACTOR |  |  |  |  |  |  |  | .969 |
|  | Freq. of rework & defects | .037 | .133 | -.082 | .068 | .011 | .906 |  |
|  | Innovation | .000 | .128 | -.066 | .061 | .041 | .902 |  |
|  | Technology capability | .048 | .107 | -.059 | .016 | .000 | .891 |  |
|  | Manager and staff competence | .086 | .103 | -.081 | .061 | .040 | .871 |  |

The commonalities of all KPIs are higher than the cutoff level adopted for this study. In addition, all KPIs show clean loadings in the new factor structure extracted. Hence, the 53 KPIs can be grouped into 6 principal components of finance (firm and market) factor, environmental factor, procurement factor, project management factor, business procedure factor and organisational knowledge factor.

  - Ranking of Measurement Factors for KPIs

The mean and standard deviations of the factors were investigated to create a ranking structure. In addition, similarities in the ranking was investigated between the three strata of Lagos, Abuja and Rivers. Results in table 5 below show a similarity in the perception of the respondents in the ranking of the factors.

Table 5:   Results of Ranking of the Measurable Factors

| Strata | All 3 Strata |  |  | Lagos |  | Abuja |  |  |  | Rivers |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | N = 244 |  |  | N = 97 |  | N = 59 |  |  |  | N = 88 |  |  |
| Factor Groups | Mean | SD | Rank | Mean | SD | Rank | Mean | SD | Rank | Mean | SD | Rank |
| Procurement Factors | 5.983 | 0.809 | 1 | 6.021 | 0.771 | 1 | 6.034 | 1.241 | 1 | 5.906 | 0.864 | 2 |
| Business Procedure Factors | 5.948 | 0.824 | 2 | 5.977 | 0.868 | 2 | 5.962 | 0.832 | 2 | 5.910 | 0.770 | 1 |
| Financial Factors | 5.792 | 0.937 | 3 | 5.946 | 0.884 | 4 | 5.372 | 1.101 | 5 | 5.903 | 0.784 | 3 |
| Organisational Knowledge Factors | 5.718 | 1.186 | 4 | 5.948 | 0.981 | 3 | 5.660 | 1.308 | 3 | 5.497 | 1.268 | 5 |
| Project Management Factors | 5.434 | 1.105 | 5 | 5.175 | 1.241 | 5 | 5.492 | 1.068 | 4 | 5.682 | 0.872 | 4 |
| Environmental Factors | 4.409 | 0.975 | 6 | 4.482 | 0.993 | 6 | 4.775 | 0.942 | 6 | 4.083 | 0.989 | 6 |

- DISCUSSION OF FINDINGS

The empirical results obtained in this study have provided useful findings and implications for the Nigerian Indigenous Contractors. Organisational performance factors can be prioritized in accordance to the factors highlighted in Table 5.

Procurement factors comprising KPIs such as procurement method, tender strategy, quality assurance requirements and effective safety programmes with factor loadings of 0.964, 0.963, 0.954 and 0.954 respectively generated an average mean of 5.98 ranking highest as factor to be measured for organisational performance of NICs. This factor is consistently ranked highest in both Lagos and Abuja and ranked second in Rivers. The standard deviation of 0.809 also shows a good internal consistency in the responses from the strata surveyed. Lagos showed the highest consistency with standard deviation of 0.77 while Rivers despite ranking the factor as second still has a standard deviation of 0.86. The KPIs measured under this factor is consistent with the previous findings of Ali et al., (2013) that posited procurement methods and tender strategy as internal business factors and Alias et al., (2014) positing quality assurance requirements and effective safety programmes as project procedure factors. Similarly, Wibowo et al., (2015) highlighted the importance of bidding strategy to the overall performance of a construction firm. The bidding strategy is posited to contribute largely to a firm’s success in securing new projects and ensuring competitiveness.

The importance of how construction firms do business was revealed as a factor worthy of measuring organisational performance in this study. Identifying KPIs such as sub-contractor management, resource management, planning efforts/effects and business efficiency all with factor loadings above 0.70. Business procedure factors ranked second in the 6 factors revealed in this study. Similarly, Lagos and Abuja strata equally ranked it second while in this case Rivers ranked it as first. Alias et al., (2014) equally identified these constructs as Critical Success Factors (CSF) from the perspective of project management practitioners looking at organisational performance of construction firms in projects. Measuring these as KPIs is seen by respondents as important to the organisational performance of construction firms.

It is surprising to find that finance factors though having the largest measurable KPIs (20) ranked third overall in the position of KPIs measured in this study despite a factor loading ranging from 0.726 – 0.968 on the KPIs. Bearing in mind the pivotal nature of financial aspects of running a business, procurement and business procedure factors ranked higher in this regard. This finding supports Hidiroglu, (2019) who strongly posited that organisational performance measurement has been identified to be a priority to project success and it should be viewed in both financial and non-financial dimensions. With an average mean of 5.792 and a standard deviation of 0.937, the factor is equally worthy of note. Also surprising is that Lagos and Abuja ranked it fourth and fifth respectively, though the standard deviation in Abuja suggest an inconsistency in the responses, Rivers on the other hand showed a standard deviation of 0.78 which shows fair consistency in the responses and also ranked it third.

Organisational Knowledge factor comprising KPIs such as frequency of rework, innovations, technology capability and manager/staff competence with factor loadings ranging from 0.871 – 0.906 was ranked fourth overall with a mean of 5.718. The internal consistency of the responses however seem low. Despite the overall results, Lagos and Abuja both ranked it as third while Rivers ranked it fifth. Similarly project management factors ranked fifth with a mean of 5.434 with weak consistency in the responses at a standard deviation of 1.105. Lagos ranked this factor fifth while both Abuja and Rivers ranked it fourth.  .

Environmental factors ranked last in the study. This is equally consistent with all strata sampled. The mean for the factor is 4.409 and standard deviation showed an equally fair consistency < 1. KPIs measured under these factor include impact on society, fulfilling government policies, emission and waste project nature, amongst others. The consistency in ranking this factor as last could be a result of the fact that strict environmental laws are not firmly in place in the country.

- CONCLUSION

This research has presented a comprehensive and systematic study on the KPIs for measuring organisational performance of NICs in the Nigerian Construction Industry. It has advanced the knowledge in the field by identifying and subsequently ranking the important KPIs for measuring performance factors in Nigeria. The study has revealed that Key Performance Indicators measurable at the organisational level of the Nigerian Indigenous contractor exist. Fifty-three KPIs were identified in this study. These KPIs measure 6 factors of finance (firm and market) factor (20 KPIs), environmental factor (16KPIs), procurement factor (4 KPIs), project management factor (5 KPIs), business procedure factor (4 KPIs) and organisational knowledge factor (4 KPIs). Five of these factors are significant with mean scores above 5. They include procurement factor ranked first, business procedure factor ranked second, finance factor ranked third, organisational knowledge factor ranked fourth and project management factor ranked fifth. The sixth factor is environmental factor which pooled a mean below 5.

The results from this study would help NICs in properly measuring their performance at the organisational level thereby improving their competitive advantage and enhancing their project delivery. Also the research study will help agencies in government sector and other construction clients in developing a metrics for evaluating the organisational performance of potential contracting firms to be employed for construction works.

One of the limitations of this research is that the samples were taken from only three strata, though representing regions of high construction activity in the country but further research could be conducted to test the KPIs in other areas. Although the statistical analysis has indicated that the findings are valid and can be generalized to a wider context, it would have been better if more samples from different strata could be acquired. Another direction to further this study is the development of a framework for the measurement of the KPIs to be adopted by NICs in particular and the construction industry in general.

- REFERENCES:

Adams, O. (1997). Contractor development in Nigeria: perceptions of contractors and professionals. Construction Management and Economics, 5(1), 95-108.

Afzal F., Lim B., and Prasad D. (2017). An investigation of corporate approaches to sustainability in the construction industry. Procedia Engineering, 180 (2017). 202 – 210.

Ali H. M., Al-Sulaihi I. A., and Al-Gahtani K. S. (2013), Indicators for measuring performance of building construction companies in Kingdom of Saudi Arabia. Journal of King Saud University – Engineering Science, 1 (2013), 125 – 134.

Alias Z., Zawawi E. M. A., Yusof K., and Aris, N. M. (2012). Determining critical success factors of project management. Procedia – Social and Behavorial Sciences. 153 (2014). 61 – 69

Antunes da Silva F. and Borsato M. (2017). Organizational Performance and Indicators: Trends and Opportunities. 27th International Conference on Flexible Automation and Intelligent Manufacturing (FAIM2017), 27-30 June 2017, Modena, Italy. Procedia Manufacturing. 11 (2017). 1925 – 1932

Chan, A.P.C. and Chan, A.P.L., (2004). Key performance indicators for measuring construction success. An International Journal, 11 (2), 203-221.

Chen, C., Chiu, P., Orr, R. and Goldstein A. (2007). An Empirical Analysis of Chinese Construction Firms’ Entry into Africa. International Symposium on Advancement of Construction Management and Real Estate, Sydney Australia, August 8th -13th  2007.

Enshassi A., Mohamed S., and Abushaban S. (2009). Factors affecting the performance of construction projects in the Gaza strip. Journal of Civil Engineering and Management, 15 (3), 269-280

Fernanda A. and Milton B. (2017). Organizational performance and indicators: Trends and opportunities. Procedia Manufacturing. 11(2017) 1925 – 1932.

Field, A. (2005). Discovering statistics using SPSS, London; Thousand Oaks, CA: Sage

Publications.

Gaffney Owen (2014). Sustainable Development Goals: Improving human and planetary wellbeing. Global Change. 82 (2014). 20 – 23.

Goodenough D. O., Albert P. C. Chan, and Ayirebi D. (2017). A review of stakeholder management performance attributes in construction projects.  International Journal of Project Management, 25 (6), 1037 - 1051.

Gudiene N., Banaitis A., Banaitiene N., and Lopes J. (2013). Development of a conceptual critical success Factors Model for construction projects: a case study of Lithuania. Procedia Engineering, 57 ( 2013), 392 – 397.

Hair, J., Black, W., Babin, B., Anderson, R., and Tatham, R. (2006). Multivariate data analysis (6th ed). Upper Saddle River, NJ: Pearson Education International-Prentice Hall.

Hidiroglu D. (2019). Self- assessment Performance Measurement in Construction Companies: An Application of the EFQM Excellence Model on Processes and Customer Stages. Procedia Computer Science 158 (2019), 844–851

Idoro G. (2009), Comparing levels of use of project plans and performance of traditional contract and design‐build construction projects in Nigeria. Journal of Engineering, Design and Technology, (x), 45-67.

Inuwa I. I., Daniel, S. and Ahmad, A. (2014), Investigating Nigerian Indigenous Contractors Project Planning In Construction Procurement: An Explanatory Approach. International Journal of Civil & Environmental Engineering IJCEE-IJENS, 14 (4), 16 - 25.

Kalton G. (2001), Practical methods for sampling rare and mobile populations. Proceedings of the Annual Meeting of the American Statistical Association, Maryland USA, August 5th – 9th 2001.

Konstantinos I. V., Odysseeus G. M., and Dimitrios G. M. (2012), Project performance indicators as an innovative tool for identifying sustainability perspective in green public procurement. Procedia Economics and Finance, 1 (2012), 401 – 410.

Landstrom A., Almstrom P., Winroth M., Andersson C., Oberg A. E., Kurdve M., Shahbaze S., Wiktorsson M., Windmark C., and Zackrisson M. (2018). A life cycle approach to business performance measurement systems. Procedia Manufacturing, 27 (2018). 126 – 133.

Lin, G., Shen, G. Q., Sun, M., and Kelly, J. (2011). Identification of key performance indicators for measuring the performance of value management studies in construction. Journal of Construction Engineering and Management, ASCE, 137 (9), 698 – 706,

Mohamad H. H., Ibrahim A. H., and Massoud H. H. (2013). Assessment of the expected construction company’s net profit using neural network and multiple regression models.  Ain Shams Engineering Journal. 4 (2013), 375 – 385.

Ogbebor, P.O. (2014). Enhancing indigenous construction industry as a national goal in Nigerian development, In Akintunde, I. Nigerian Construction Industry: Past, Present, Problems and Prospects.  Ibadan,

Osborn D., Cutter A. and Ullah F. (2015). Universal Sustainable Goals – Understanding the transformational challenge for developing countries. Retrieved November 16, 2019, from Stakeholder Forum, http://www.stakeholdersforum.org/report_of_study/2015/

Sanusi A. (2012).	Construction importance in a developing economy. ASCE Journal of Construction Engineering and Management, 3 (111).190-207.

Sohail M. and Baldwin A. N. (2004), Performance indicators for micro projects in developing countries, Construction Management and Economics. 22(1), 11 - 23.

Sturgis P. (2004). Social Research Update. Retrieved December 25, 2019, from University of Surrey. Uk, http://www.epubssurrey.ac.uk/1651/1/.

Taber K. S. (2018). The use of Cronbach’s Alpha when developing and reporting research instruments in science education. Research in Science Education, DOI 10.1007/s11165-016-9602-2, 1273 – 1296.

Toor S and Ogunlana S. O., (2010). Beyond the ‘iron triangle’: Stakeholder perception of key performance indicators (KPIs) for large-scale public sector development projects. International Journal of Project Management, 28 (3), 228 - 236.

Tripathi K. K. and Jha K. N. (2017). Empirical Study on Performance Measurement Factors for Construction Organizations. KSCE Journal of Civil Engineering, 22(4).1052-1066

Wibowo M. A, Astana I. N. Y and Rusdi H. A., (2015). An analysis of bidding strategy, project performance and company performance relationship in construction. Procedia Engineering. 125 (2015). 95 – 102

Explorable.com (2009). Stratified Sampling Method. Retrieved December 24, 2019 from Explorable.com: https://explorable.com/stratified-sampling