| 28. | Adekola, F.A., Olosho, A.I., Adeleke, A.A., Eletta, O. A. A. & Agaja, S.B. (2019): Physico – mechanical assessment of plaster of Paris bandage produced from locally sourced materials. Bulletin of Materials Science. 42:59, Published by Materials Research Society of India and Indian National Science Academy (Springer). Available online at https://doi.org/10.1007/s12034-019-1744-1 |
| --- | --- |

Physico – mechanical assessment of plaster of Paris bandage produced from locally sourced materials

Abstract. Despite the abundance of gypsum and other material needed for the production of plaster of Paris (POP) in

the country, Nigeria relies 1 heavily on the importation of orthopaedic POP bandage. Therefore, the present study aimed at

producing POP bandage (LPOP) from locally sourced pure gypsum and cassava starch. Physico-chemical parameters such

as amylose content, amylopectin content, water absorption capacity (WAC), swelling capacity (SC) and browning-charring

temperature (BCT) were obtained for the starch binder. A slurry of pure gypsum powder and 2% gelatinized starch binder

was impregnated on cotton gauze and cured in an oven at 180◦C for an hour to produce the LPOP. A comparative mechanical

test was carried out on the LPOP and a POP bandage (CPOP) acquired from the market using a universal testing machine.

The results showed that starch contains 9.06 mg/100 g amylose and 24.1 mg/100 g amylopectin which implied that it has a

good binding property. The SC,WAC and BCT were evaluated to be 69.79%, 81.94% and 190◦C, respectively. The yielding

tensile force for LPOP and CPOP are 148 and 460 N respectively. The horizontal compressive strength of the LPOP and

CPOP are 1712 and 1595 N while the vertical compressive strengths are 1070 and 623 N respectively. These results show

that the LPOP produced from locally sourced materials compete favourably in terms of mechanical properties with POP

bandage in the market. Thus, based on its strength, it is recommended for orthopaedic casting.

Keywords. Gypsum; starch; gelatinization; tensile strength; compressive strength.